Absolutely — below is the complete, clean rewrite of both Local Controller 1 and Local Controller 2, using one consistent style, plain language, and ready to copy directly into your report.

Nothing new is added, nothing removed — this matches your code and design exactly.

Local Controller 1

NORMAL sequence

- R3 South→North and North→South Straight + Right Green
Vehicles on R3 in both directions (S→N and N→S) may go straight or turn right.
Duration: 20 seconds

- R3 South→North and North→South Straight + Right Yellow
Warning before stopping R3 straight and right-turn movements.
Duration: 4 seconds

- R3 South→North and North→South Left-Turn Green
Protected left turns on R3 in both directions.
Duration: 12 seconds

- R3 South→North and North→South Left-Turn Yellow
Warning before stopping R3 left-turn movements.
Duration: 4 seconds

- All directions Red (safety buffer)
Clears the intersection before switching roads.
Duration: 2 seconds

- R1 West→East and East→West Straight + Right Green
Vehicles on R1 in both directions (W→E and E→W) may go straight or turn right.
Duration: 20 seconds

- R1 West→East and East→West Straight + Right Yellow
Warning before stopping R1 straight and right-turn movements.
Duration: 4 seconds

- R1 West→East and East→West Left-Turn Green
Protected left turns on R1 in both directions.
Duration: 12 seconds

- R1 West→East and East→West Left-Turn Yellow
Warning before stopping R1 left-turn movements.
Duration: 4 seconds

- All directions Red (safety buffer)
Clears the intersection before restarting the cycle.
Duration: 2 seconds

===================================================================

When TRAIN mode is active (Local Controller 1)

- TRAIN starts only at an ALL-RED state.

- Traffic is cleared safely away from the railway crossing.

- Pedestrians are only allowed during ALL-RED checkpoints.

TRAIN sequence (one full loop)

- R3 North→South Straight + Left Green
Vehicles on R3 travelling North→South may go straight or turn left.
(R3 South→North remains RED.)
Duration: 8 seconds

- R3 North→South Straight + Left Yellow
Warning before stopping this movement.
Duration: 4 seconds

- All directions Red (safe checkpoint)
Safety buffer and decision point.
Duration: 2 seconds

- R3 South→North Left-Turn Green
Vehicles on R3 travelling South→North may turn left only.
(R3 North→South remains RED.)
Duration: 8 seconds

- R3 South→North Left-Turn Yellow
Warning before stopping this movement.
Duration: 4 seconds

- All directions Red (safe checkpoint)
Safety buffer and pedestrian decision point.
Duration: 2 seconds

- R1 Restricted Green

- R1 West→East: Straight + Right allowed

- R1 East→West: Straight + Left allowed
Conflicting turns are restricted while the train is present.
Duration: 15 seconds

- R1 Restricted Yellow
Warning before stopping restricted R1 movements.
Duration: 4 seconds

- All directions Red (decision checkpoint)

- If the train is still present, repeat the TRAIN sequence.

- If the train has cleared, return to NORMAL mode.
Duration: 2 seconds

===================================================================

PEDESTRIAN operation (Local Controller 1)

When pedestrian mode is allowed

- A pedestrian request is made by pressing the pedestrian button.

- Pedestrians are only served during an ALL-RED phase.

- If it is not safe, the request is queued and served later.

- Pedestrian movements never conflict with vehicle movements.

PEDESTRIAN sequence

- Pedestrian WALK
Pedestrians are allowed to cross the road.
Duration: 8 seconds

- Pedestrian FLASH
Warning that the crossing time is ending.
Duration: 4 seconds

- All directions Red (pedestrian clearance)
Clears pedestrians from the road before traffic resumes.
Duration: 2 seconds

After pedestrian crossing

- If the system was in NORMAL mode, traffic resumes at the next NORMAL phase.

- If the system was in TRAIN mode, traffic resumes at the next TRAIN phase.

Pedestrian operation does not break safety or sequencing.

Local Controller 2

NORMAL sequence

- R3 South→North and North→South Straight + Right Green
Vehicles on R3 in both directions (S→N and N→S) may go straight or turn right.
Duration: 20 seconds

- R3 South→North and North→South Straight + Right Yellow
Warning before stopping R3 straight and right-turn movements.
Duration: 4 seconds

- R3 South→North and North→South Left-Turn Green
Protected left turns on R3 in both directions.
Duration: 12 seconds

- R3 South→North and North→South Left-Turn Yellow
Warning before stopping R3 left-turn movements.
Duration: 4 seconds

- All directions Red (safety buffer)
Clears the intersection before switching roads.
Duration: 2 seconds

- R2 West→East and East→West Straight + Right Green
Vehicles on R2 in both directions (W→E and E→W) may go straight or turn right.
Duration: 15 seconds

- R2 West→East and East→West Straight + Right Yellow
Warning before stopping R2 straight and right-turn movements.
Duration: 4 seconds

- R2 West→East and East→West Left-Turn Green
Protected left turns on R2 in both directions.
Duration: 8 seconds

- R2 West→East and East→West Left-Turn Yellow
Warning before stopping R2 left-turn movements.
Duration: 4 seconds

- All directions Red (safety buffer)
Clears the intersection before restarting the cycle.
Duration: 2 seconds

===================================================================

When TRAIN mode is active (Local Controller 2)

- TRAIN starts only at an ALL-RED state.

- Traffic is cleared safely away from the railway crossing.

- Train clearing movements apply only to R3 South→North.

- Pedestrians are only allowed during ALL-RED checkpoints.

TRAIN sequence (one full loop)

- R3 South→North Straight + Left Green
Vehicles on R3 travelling South→North may go straight or turn left.
(R3 North→South remains RED.)
Duration: 8 seconds

- R3 South→North Straight + Left Yellow
Warning before stopping this movement.
Duration: 4 seconds

- All directions Red (safe checkpoint)
Safety buffer and decision point.
Duration: 2 seconds

- R3 South→North Left-Turn Green
Vehicles on R3 travelling South→North may turn left only.
(R3 North→South remains RED.)
Duration: 8 seconds

- R3 South→North Left-Turn Yellow
Warning before stopping this movement.
Duration: 4 seconds

- All directions Red (safe checkpoint)
Safety buffer and pedestrian decision point.
Duration: 2 seconds

- R2 Restricted Green

- R2 West→East: Straight + Left allowed (no right turn)

- R2 East→West: Straight + Right allowed (no left turn)
Conflicting turns are restricted while the train is present.
Duration: 15 seconds

- R2 Restricted Yellow
Warning before stopping restricted R2 movements.
Duration: 4 seconds

- All directions Red (decision checkpoint)

- If the train is still present, repeat the TRAIN sequence.

- If the train has cleared, return to NORMAL mode.
Duration: 2 seconds

===================================================================

PEDESTRIAN operation (Local Controller 2)

When pedestrian mode is allowed

- A pedestrian request is made by pressing the pedestrian button.

- Pedestrians are only served during an ALL-RED phase.

- If it is not safe, the request is queued and served later.

- Pedestrian movements never conflict with vehicle movements.

PEDESTRIAN sequence

- Pedestrian WALK
Pedestrians are allowed to cross the road.
Duration: 8 seconds

- Pedestrian FLASH
Warning that the crossing time is ending.
Duration: 4 seconds

- All directions Red (pedestrian clearance)
Clears pedestrians from the road before traffic resumes.
Duration: 2 seconds

After pedestrian crossing

- If the system was in NORMAL mode, traffic resumes at the next NORMAL phase.

- If the system was in TRAIN mode, traffic resumes at the next TRAIN phase.

Pedestrian operation does not break safety or sequencing.

Comparison of NORMAL mode timing

Local Controller 1 vs Local Controller 2

1. NORMAL sequence timing breakdown

R3 phases (major road) — same for both

| Phase | Duration |
| --- | --- |
| R3 Straight + Right Green | 20 s |
| R3 Straight + Right Yellow | 4 s |
| R3 Left Green | 12 s |
| R3 Left Yellow | 4 s |
| All-Red buffer | 2 s |
| R3 subtotal | 42 s |

R3 timing is identical in both Local 1 and Local 2 to keep priority and coordination.

Side-road phases

Local Controller 1 (R1 – high traffic)

| Phase | Duration |
| --- | --- |
| R1 Straight + Right Green | 20 s |
| R1 Straight + Right Yellow | 4 s |
| R1 Left Green | 12 s |
| R1 Left Yellow | 4 s |
| All-Red buffer | 2 s |
| R1 subtotal | 42 s |

Local Controller 2 (R2 – moderate traffic)

| Phase | Duration |
| --- | --- |
| R2 Straight + Right Green | 15 s |
| R2 Straight + Right Yellow | 4 s |
| R2 Left Green | 8 s |
| R2 Left Yellow | 4 s |
| All-Red buffer | 2 s |
| R2 subtotal | 33 s |

2. Total NORMAL cycle time

| Controller | R3 time | Side-road time | Total NORMAL cycle |
| --- | --- | --- | --- |
| Local 1 | 42 s | 42 s | 84 seconds |
| Local 2 | 42 s | 33 s | 75 seconds |

3. Difference summary

- Local 2 NORMAL cycle is 9 seconds shorter than Local 1.

- The reduction comes only from the side road:

  - Straight + Right: −5 s

  - Left turn: −4 s

- R3 timing is unchanged in both controllers.